TXT70~TXT80 (Rookie Trial)

Transmission lubricant.

Oil capacity of transmission is 650cc 5W30 non-synthetic motor oil. This motorcycle uses a “dipstick type” oil filler cap. Check oil level of the transmission with dipstick resting in hole in un-screwed position.

[image: image1.jpg]



PRIVATE
ENGINE

Two stroke single cylinder


PRIVATE

CYLINDER

chrome with 6 ports


PRIVATE

INTAKE

4 petal case reed valve


PRIVATE

BORE

47,0 mm 


PRIVATE

STROKE 

41,8 mm 


PRIVATE

DISPLACEMENT 

69 cc 


PRIVATE

COMPRESSION RATIO 

11,7 +/- 0,2


PRIVATE

MAX POWER 

8,2 HP (8,0 KW) at 8500 rpm


PRIVATE

MAX TORQUE 

7,0 Nm at 8500 rpm



PRIVATE

COOLING

Mechanical water pump with thermostat controlled electric cooling fan motor.


PRIVATE

FUEL

50:1 Gasoline and oil (premix)


PRIVATE

STARTING

“Primary” Kickstarter 


PRIVATE

CARBURETOR 

Dell'Orto PHBG21DS


PRIVATE

GENERATOR

magneto flywheel 85W 12V


PRIVATE

IGNITION

External H.V. 


PRIVATE

SPARK PLUG 

NGK BP5ES BPR5ES 

Champion RN11YC


PRIVATE

PRIMARY GEARING 

gears 21/74


PRIVATE

SECONDARY DRIVE

By chain 1/2" x 3/16"


PRIVATE

CLUTCH 

Hydraulically operated, in oil bath, with multiple discs


PRIVATE

GEAR BOX 

6 speed



ENGINE RATIOS

1st gear = 1 : 12,814
2nd gear = 1 : 8,692
3rd gear = 1 : 6,656
4th gear = 1 : 5,286
5th gear = 1 : 4,290
6th gear = 1 : 3,806








PRIVATE

OIL CAPACITY 

600cc SAE 5W/30~10W/40


PRIVATE

ENGINE DRY WEIGHT 

Kg. 13,500  (29.7 lbs.)

Carburetor set-up options. (Hints)

1. Needle in full rich position. (needle clip in bottom slot position)

2. Cut angle on SLIDE within 2mm of the bottom deck. Note: There is a PDF adobe file with instructions for this modification. 

3. PHBG21DS carburetor specifications:    Main jet: 92 ~ Pilot jet: 50 ~ Needle jet: 262AU ~ Needle W7 in top clip position. ~ Slide: nr.40 (modified) ~ 

4. As per Tony Bussing at the Trials Training Center, Sequatchie Tennessee. 
 (NOTE: Not tested by GASGAS USA. Not an official GASGAS recommendation, but worthy of mention.)

“So far, this is what I have done to the 70's”:


A. Needle in full rich position
B. Cut angle on SLIDE within 2mm of the bottom deck
C. Tape up smaller half of airbox opening (exactly half of opening)


These modifications take away some of the hit and make it grunt.

Tony Bussing


Chassis capacities and specifications.

NOTE: Capacities and specifications for chassis components are the same as the full size GasGas trial bikes.







